Tarun kumar R

25MML0057

Edge-intelligence

Canny Equation

Linear transformation →

S = (L − 1) − r

S : the output pixel intensity (negative)
r : the input pixel intensity (original)

L : no of possible intensity levels
(8-bit → L = 256)

⇒ Iₒᵤₜ = 255 − Iᵢₙ

0 (black) ⇒ 255 − 0 = 255 (white)

255 (white), the new
255 − 255 = 0 (black)

mid tones ⇒ 100 → 155 {255 − 100 = 155}

The Canny Edge Detection method does not have a single equation; it is a sequence of mathematical steps. The key equations used in the algorithm are below.

1. Gaussian Smoothing (Noise Reduction)

The image is first smoothed using a Gaussian filter:

G(x,y)=12πσ2e-x2+y22σ2

Smoothed image:

Is(x,y)=I(x,y)*G(x,y)

Where:

- I(x,y)= original image

- G(x,y)= Gaussian kernel

- *= convolution

- σ= standard deviation of Gaussian

2. Gradient Calculation

The intensity gradients in x and y directions are computed:

Gx=∂I∂x,Gy=∂I∂y

Gradient magnitude:

G=Gx2+Gy2

Gradient direction:

θ=tan⁡-1GyGx

3. Non-Maximum Suppression

Edges are thinned by keeping only local maxima:

G(x,y)=G(x,y),if local maximum0,otherwise

4. Double Thresholding

Two thresholds are applied:

G(x,y)=Strong edge,G≥THWeak edge,TL≤G<TH0,G<TL

Where:

- TH= high threshold

- TL= low threshold

5. Edge Tracking by Hysteresis

Weak edges are kept only if connected to strong edges.

Key equation often written in exams:

G=Gx2+Gy2,θ=tan⁡-1GyGx